HISPAMAR SATÉLITES S. A.

AMAZONAS-3 SATELLITE TECHNICAL DESCRIPTION (KA BAND)

A.1 GENERAL DESCRIPTION

The purpose of this technical attachment is to describe the satellite system in particular in the Ka band for the AMAZONAS-3 satellite.

The AMAZONAS-3 satellite is foreseen to be launched on 4rd Q. 2012 and be operated at the 61 W.L. orbital location.

The AMAZONAS-3 satellite will be a replacement of the AMAZONAS-1 satellite, located in the same orbital location. The AMAZONAS-3 will keep the same C and Ku frequency bands and will add Ka frequency band.

The Ka band system will be composed by the Forward Link and the Return Link. In the Forward Link the sense of transmission will be:

Gateway terminal -> Satellite -> User Terminal

While in the Return Link the sense of transmission will be:

User Terminal -> Satellite -> Gateway terminal.

Figure 1: Forward Link Scheme

Figure 2: Return Link Scheme

There will be 3 Gateway terminals, 2 of them will be placed in the USA’s territory (Laredo and Tucson) and the other one in Chile (Arica). However, there will be only 2 Gateways working at the same time due to the Gateways in Tucson and Arica can not be operated simultaneously.

All the user terminals will be located in the cities of Mexico, Bogota, Santiago, Brasilia, Sao Paulo, Rio de Janeiro, Lima and Buenos Aires.

In general, the satellite will use for the FORWARD link the frequency 28.1 – 30.0 GHz band for Earth-to-space transmission and 19.7 – 20.2 GHz band for space-to-Earth transmission. And it will use for the RETURN link the frequency 29.5 – 30.0 GHz band for Earth-to-space transmission and 18.3 – 20.2 GHz band for space-to-Earth transmission. In the territory of the USA, the Amazonas-3 satellite will operate in the frequencies 28.1 – 28.6 GHz  and 29.5 – 30.0 GHz for Earth-to-space transmission and 18.3 – 18.8 GHz  and 19.7 – 20.2 GHz for space-to-Earth transmission. Therefore, Hispamar seeks authority to use spectrum in the 28.1-28.6 GHz, 29.5-30.0 GHz, 18.3-18.8 GHz and 19.7-20.2 GHz ranges, to support gateway operations for AMAZONAS-3.

A.2 ORBITAL LOCATION

The AMAZONAS-3 satellite will be a replacement of the AMAZONAS-1 satellite, located in the orbital location 61ºW. The AMAZONAS-3 will keep the same C and Ku frequency bands and will add Ka frequency band.

. Besides, it provides high elevation angles to all South America, which is very important for satellite services to large numbers of small and inexpensive consumer earth stations. The high elevation minimizes the risk of signal blockage due to buildings and foliage, and also minimizes the atmospheric and rain attenuation, a very important consideration for the Ka-band frequencies.

A.3 SATELLITE COVERAGE

3 Spot beams will cover the cities where the Gateways will be located:

- Laredo (USA).

- Tucson (USA).

- Arica (Chile).

8 Spot beams will cover the cities where the Users will be located:

- Mexico (Mexico).

- Bogota (Colombia).

- Santiago (Chile).

- Brasilia (Brazil).

- Sao Paulo (Brazil).

- Rio de Janeiro (Brazil).

- Lima (Peru).

- Buenos Aires (Argentina).

In the figure 3 in section A.5 are represented all the spots beams.

A.4 FREQUENCY AND POLARIZATION PLANS

The frequency, polarization and beam connectivity scheme for the Amazonas-3 satellite’s

communications transponders in Ka band, operating in the USA’s territory, is provided in Table 1.

|  |  | FREQUENCY (MHz) |  | POLARIZATION |  | COVERAGE |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TRANSPONDER | BW(MHZ) | UPLINK | DOWNLINK | UPLINK | DOWNLINK | UPLINK | DOWNLINK |
| 1 | 450 | 28350 | 19950 | RHCP | RHCP | Laredo USA | Mexico MEX |
| 2 | 450 | 28350 | 19950 | LHCP | LHCP | Laredo USA | Mexico MEX |
| 3 | 450 | 29750 | 19950 | RHCP | RHCP | Laredo USA | B. Aires ARG |
| 4 | 450 | 29750 | 19950 | LHCP | LHCP | Laredo USA | Rio BR |
| 5 | 450 | 28350 | 19950 | RHCP | RHCP | Tucson USA/Arica CHL | Sao Paulo BR |
| 6 | 450 | 28350 | 19950 | LHCP | LHCP | Tucson USA/Arica CHL | Santiago CHL |
| 7 | 450 | 29750 | 19950 | RHCP | RHCP | Tucson USA/Arica CHL | Bogota CLM |
| 8 | 450 | 29750 | 19950 | LHCP | RHCP | Tucson USA/Arica CHL | Lima PRU |
| 9 | 450 | 29750 | 18550 | LHCP | LHCP | Mexico MEX | Laredo USA |
| 10 | 450 | 29750 | 18550 | RHCP | RHCP | Mexico MEX | Laredo USA |
| 11 | 450 | 29750 | 19950 | LHCP | LHCP | B. Aires ARG | Laredo USA |
| 12 | 450 | 29750 | 19950 | RHCP | RHCP | Rio BR | Laredo USA |
| 13 | 450 | 29750 | 18550 | LHCP | LHCP | Sao Paulo BR | Tucson USA/Arica CHL |
| 14 | 450 | 29750 | 18550 | RHCP | RHCP | Santiago CHL | Tucson USA/Arica CHL |
| 15 | 450 | 29750 | 19950 | LHCP | LHCP | Bogota CLM | Tucson USA/Arica CHL |
| 16 | 450 | 29750 | 19950 | LHCP | RHCP | Lima PRU | Tucson USA/Arica CHL |

NOTES: BR = Brazil / MEX = Mexico / ARG = Argentina / CHL = Chile / CLM = Colombia / PRU = Peru

Table 1.- AMAZONAS-3 Ka-band Frequency Plan Definition. USA transponders.

A.5 SATELLITE TRANSMIT CAPABILITY

The AMAZONAS-3 satellite in Ka band is composed by 11 spot beams, focused in the cities detailed in section A.3 and represented in figure 3. Each of them has a peak antenna gain of 48 dBi and 43dBi in the edge. The TWTAs have an effective output power of 17 dBW and the losses between the DTWTA output and the antenna input amount to 2.5 dB. It results a maximum EIRP of 65 dBW. The cross-polar isolation of each spot beam will exceed 26 dB. In figure 3 are represented the coverage of all spot beams.

The system is designed to transmit in both RHC and LHC polarizations on the same frequency, meeting the requirements for frequency re-use of the spectrum as required by Section 25.210(d) of the Commission’s Rules.

Figure 3: Amazonas-3 Ka spot beams.

Contours of these beams are included in the Schedules S.

A.6 SATELLITE RECEIVE CAPABILITY

The AMAZONAS-3 satellite in Ka band is composed by 11 spot beams, focused in the cities detailed in section A.3 and represented in figure 3. All the spot beams are nominally identical, each with a peak gain of 52 dBi. The receive system noise temperature is 2570 K and the beam peak G/T performance is +17.9 dB/K. The crosspolar isolation of the satellite receive antennas will exceed 26 dB.

The uplink spot beams on the Amazonas-3 satellite operate using LHCP and RHCP.

Contours of these beams are included in the Schedules S.

A.7 TRANSPONDER GAIN CONTROL AND SATURATION FLUX DENSITY

All Ka band transponders will be operated in an adjustable gain mode which is programmable by ground command across a 20 dB range in 1 dB steps.

The minimum SFD and maximum Ka transponder gains for each beam type are included in the Schedule S form. The SFDs and transponders gains vary over the 20 dB gain range.

A.8 SATELLITE TRANSPONDER FILTER RESPONSE

The specification for the overall Ka transponder in-band filter response and out-of-band attenuation is dictated by the following considerations:

- The in-band gain and group delay response must be flat enough so as not to degrade significantly the bit error rate performance of the digital carrier(s) in the transponder;

- The out-of-band attenuation must be high enough, in the adjacent transponder frequency band, to suppress adequately the multi-path transmission through adjacent transponders.

- The out-of-band attenuation must also be sufficient to suppress any unwanted signals infrequency bands adjacent to the transponder frequency band, which could otherwise cause overload of the active amplifiers in the communications payload, or waste the available power of the TWTAs.

| % OF CHANNEL BANDWIDTH | 55% | 85% | 90% | 100% |
| --- | --- | --- | --- | --- |
| INPUT SECTION GAIN FLATNESS dBpp | 0.65 | 0.7 | 1.2 | 2.3 |
| TOTAL GAIN FLATNESS dBpp | 0.85 | 1.3 | 2.4 | 4.6 |
| INPUT SECTION GAIN SLOPE dB/MHz | 0.15 | 0.2 | 0.5 | 1.3 |
| TOTAL GAIN SLOPE dB/MHz | 0.25 | 0.4 | 1.0 | 2.9 |

Table 2.- Amplitude in Ka-band response

A.9 UNWANTED EMISSIONS

The out-of-band emissions will not exceed the limits of §25.202(f) (1), (2) and (3).

A.10 EMISSION DESIGNATORS AND ALLOCATED BANDWIDTH OF EMISSIONS

The allocated bandwidths and emission designators used by the Amazonas-3 system in the Ka band will be in the range between 1 MHz / 1M00G7W and 450 MHz / 450MG7W.

A.11 EARTH STATIONS

Terminal users Ka-band transmit/receive earth station to be used with the Amazonas-3

satellite will be a 75 cm antenna. Such terminals are expected to be deployed in large numbers across the service areas (several millions). In some areas and for certain applications, where higher clear-sky performance is required, larger antennas may be used (typically 90 cm, 120 cm, or 150 cm).

In addition there will be a small number of large gateway earth stations that are used for aggregating and uplinking local programming and transporting signals back to the terrestrial networks, such as for Internet connections. These gateway stations will typically use an antenna of 6 meters.

A.12 LINK BUDGETS

The documents Forward Ka-band 1.pdf, Forward Ka-band 2.pdf, Forward Ka-band 3.pdf, Forward Ka-band 4.pdf, return ka 5.pdf, return ka 6.pdf, return ka 7.pdf, return ka 8.pdf, DTH Ka-band 9.pdf  provided for the Schedule S show representative link budgets in the Ka band for some type of emission used by the Amazonas-3 system in the Ka band, including examples for the Forward link, the Return link and DTH (Direct to home) transmissions.

A.13 POWER FLUX DENSITY AT THE EARTH’S SURFACE

§25.208(c) contains FSS Power Flux Density (“PFD”) limits that apply in the 18.3-18.8 GHz band. The PFD limits of §25.208(c) are as follows:

- -115 dB (W/m2) in any 1 MHz band for angles of arrival between 0 and 5 degrees above the horizontal plane;

- -115+(d-5)/2 dB (W/m2) in any 1 MHz band for angles of arrival d (in degrees) between 5 and 25 degrees above the horizontal plane; and

- -105 dB (W/m2) in any 1 MHz band for angles of arrival between 25 and 90 degrees above the horizontal plan

In addition, §25.208(d) contains FSS Power Flux Density (“PFD”) limits that apply in the 18.6- 18.8 GHz band produced by emissions form a space station under assumed free-space propagation conditions as follows:

- -95 dB(W/m2) for all angles of arrival. This limit may be exceeded by up to 3 dB for no more than 5% of the time.

Compliance with these limits is demonstrated below using a simple worst-case methodology:

In all the Ka downlink frequency range, the power flux density at the Earth’s surface produced by emissions from a space station for all conditions and for all methods will not exceed -123.5 dBW/m2 /1 MHz over the US territory and all the Americas:

-69.5 dBW/Hz max psd + 48 (max gain dBi) - 162 spreading losses (dB) (worst case).

In addition, §25.208(d) provides an additional aggregate PFD limit in the 200 MHz wide

band 18.6-18.8 GHz of -95 dBW/m2. In the worst case this would correspond to a PFD limit per 26 MHz of -118 dBW/m2/MHz (i.e., -95-10*log(200 MHz)). Therefore, based on the worst-case calculation in the preceding paragraph, compliance with the §25.208(d) is also assured.

A.14 CESSATION OF EMISSIONS

Each satellite transponder can be individually turned on and off by ground telecommand, thereby causing cessation of emissions from the satellite, as required.

A.15 INTERFERENCE STATEMENT

Compliance with the FCC two-degree spacing policy is assured provided:

- The maximum PFD levels in the Ka band are lower than the PFD values given in §25.138 (a)(6) of the Rules for blanket licensing of small user terminals;

- The uplink off-axis EIRP density limits of §25.138 (a)(1) of the Rules are not exceeded.

Section A.13 showed the maximum PFD that can be transmitted by the Amazonas-3 in the Ka band satellite at any angle of arrival is -123.5 dBW/m2/MHz. This value is lower than the PFD value given in §25.138 (a)(6) of the Rules for blanket licensing of small user terminals.

The clear sky uplink input power density in the Ka band will be always lower than -56.5 dBW/Hz, according to the value given in §25.138 (a)(1). No authorized uplink transmissions to the Amazonas-3 satellite will exceed an input power density value of -56.5 dBW/Hz in the Ka band.

| Uplink Antenna Size | Emission | Maximum Clear Sky Uplink Input Power Density (dBW/Hz) | Clear Sky Uplink Input Power Density Limit of §25.138 (a)(1) (dBW/Hz) | Excess Margin (dB) |
| --- | --- | --- | --- | --- |
| 6 m | 450MG7W | -71.1 | -56.5 | 14.6 |
| 6 m | 45M0G7W | -67 | -56.5 | 10.5 |
| 0.75 m | 5M00G7W | -64.9 | -56.5 | 8.4 |
| 0.75 m | 1M00G7W | -58.64 | -56.5 | 2.14 |

Table 3.- Examples of demonstration of compliance with the Uplink Power limits of §25.138 (a)(1)

A.16 SHARING ANALYSIS WITH OTHER SERVICES AND ALLOCATIONS

FSS Stations licensed to use spectrum within the 18.3-18.8 GHz band are protected by virtue of the GSO FSS PFD limits contained in Section 25.208(c) of the Rules. Section A.13 (PFD) above demonstrates that the Amazonas-3 satellite in the Ka band does not exceed the PFD limits of Section 25.208(c) of the Rules.