CONSOLIDATED SCHEDULE B TECHNICAL EXHIBIT

FCC Form 312 Modification Application — Surviving Call Sign E040269

Adding the three antennas formerly authorized under expired call sign E890197, at 2000 M Street NW, Washington, DC

# 1. Filing Summary

This exhibit supports a Modification (SES-MOD) application filed against call sign E040269, adding the three antennas formerly authorized under expired call sign E890197 (A1, A2, A3) as newly proposed equipment. E040269's own four antennas (3.8M.PRO, 2.4M.COM, 4.6M.VER, 3.8M.PRO2) continue unchanged in location and technical description, except for the emission designator consolidation removing all emission types other than 36M0G7W. Upon grant, a cancellation/surrender request will be filed for E890197.

# 2. Site Information

Site: 2000 M Street NW, Washington, DC 20036 (Site ID 1). All seven antennas are located at this single rooftop site, consistent with the site data on file for both E040269 and E890197.

# 3. Consolidated Antenna / Uplink Schedule

Table reflects the Earth-to-Space frequency band per antenna, using a single 36M0G7W emission designator replacing the multiple legacy emission designators. EIRP values carry forward the previously authorized EIRP for the closest 36 MHz-class carrier on each antenna.

| Antenna ID | Mfr / Model | Size | Freq Band | Direction | Gain (dBi) | Emission | EIRP (dBW) | Source Callsign |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.8M.PRO | Prodelin 1383 | 3.8 m | 14000–14500 MHz | Earth-to-Space | 53.2 | 36M0G7W | 76.3 | E040269 |
| 2.4M.COM | Comtech 847-337-G2 | 2.4 m | 14000–14500 MHz | Earth-to-Space | 48.2 | 36M0G7W | 74.3 | E040269 |
| 4.6M.VER | Vertex 4.57KPK | 4.6 m | 14000–14500 MHz | Earth-to-Space | 54.5 | 36M0G7W | 76.67 | E040269 |
| 3.8M.PRO2 | Prodelin 1381 | 3.8 m | 14000–14500 MHz | Earth-to-Space | 53.2 | 36M0G7W | 76.3 | E040269 |
| A1 | Lucas Aerospace 2402 | 2.4 m | 14000–14500 MHz | Earth-to-Space | 49.2 | 36M0G7W | 73.2 | E890197* |
| A2 | Microdyne PR-164 | 5.0 m | 14000–14500 MHz | Earth-to-Space | 56.3 | 36M0G7W | 80.3 | E890197* |
| A3 | Satcom/RSI 500 KS | 5.0 m | 14000–14500 MHz | Earth-to-Space | 55.0 | 36M0G7W | 79.3 | E890197* |

## Downlink (Space-to-Earth) companion bands

| Antenna ID | Freq Band | Direction | Gain (dBi) | Emission | Source Callsign |
| --- | --- | --- | --- | --- | --- |
| 3.8M.PRO | 11700–12200 MHz | Space-to-Earth | 51.7 | 36M0G7W | E040269 |
| 2.4M.COM | 11700–12200 MHz | Space-to-Earth | 47.7 | 36M0G7W | E040269 |
| 4.6M.VER | 11700–12200 MHz | Space-to-Earth | 53.2 | 36M0G7W | E040269 |
| 3.8M.PRO2 | 11700–12200 MHz | Space-to-Earth | 51.7 | 36M0G7W | E040269 |
| A1 | 11700–12200 MHz | Space-to-Earth | 47.7 | 36M0G7W | E890197* |
| A2 | 11700–12200 MHz | Space-to-Earth | 53.2 | 36M0G7W | E890197* |
| A3 | 11700–12200 MHz | Space-to-Earth | 53.6 | 36M0G7W | E890197* |

# 4. Points of Communication

E040269 and E890197 both list "Permitted List / GSO" as their points of communication and can carry forward unchanged for all seven antennas.